SPECIFICATION

Structure		:RR masonry for foundation and basement with RCC belt

Wall : Laterate  masonry  with all-round RCC lintel and solid cement bricks for thin    walls with cement plastering.

Doors and Windows	: All doors and windows expect back door and Toilet doors using good quality   Jackwood or Irul wood.

Branded steel doors for back doors and Fiber doors for toilet.

Windows	: Using good quality Jack wood or Irul wood with glass with MS grill.

Painting 	: Branded Interior emulsion with putty for interior walls and branded antifungal exterior emulsion for exterior walls

Melamine polishing for wooden doors and front side.

Enamel painting for all other wooden windows.

Flooring		: Branded 60X60cm vitrified tile.

Wall tile in toilet upto 240cm height.

Kitchen dado tile 2ft.

O/H Water Tank & U/G Sump	: 2000Ltrs each.

Electrification 	: Three phase system using standard wires and pipes concealed conduit wiring     with copper conductor, adequate light, fan, telephone, TV point, 6A plug and 16 power plug etc. controlled by ELCB and MCBs.

Wires – Finolex or equalant.

Switches – Elleys or equalent modular switches.

Plumbing and Sanitation : Plumbing lines – ISI standared PVC items.

CP fitting – jaguar or Equalant.

Ceramic sanitary fixtures – cera or equalant.

Toilets – concealed plumbing with above mentioned fittings, provisions for hot and cold – water tap with shower and plumbing for heater.

(Solar water heating system – additional)

Septic Tank	: Fiber Septic Tank.

Kitchen & W/A Cabinet : SS single bowl sink and Granite top on RCC slab.

Stair case Hand Rail	:  SS Handrail.